# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Supply of Cargo communal vehicle for collection and transport of communal and recycled waste

Publication reference:

Column 1-2 should be completed by the Contracting Authority

Column 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on his proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| Item Number | Specifications |  | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committee’s notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Cargo communal vehicle for collection and transport of communal and recycled waste |  |  |  |  |
|  | Parameter | Characteristics (minimal requirements) |  |  |  |
|  | Engine type | - EURO 4 engine - 130 kW |  |  |  |
|  | Cylinder | - 4 cylinders in line |  |  |  |
|  | Total mass of the vehicle | - 11.990 kg |  |  |  |
|  | Total gross mass of empty equipped vehicle | - 8.290 kg |  |  |  |
|  | Refuse tank volume due to registration card | - 3.700 kg |  |  |  |
|  | Technical tank volume | - 4500 kg |  |  |  |
|  | Vehicle type | - cargo |  |  |  |
|  | Vehicle dimensions | - length: 7412 mm - width: 2350 mm - vehicle height: 3100 mm - height of loading space from the ground: 1400 mm |  |  |  |
|  | Seats | - 1 + 2 seats |  |  |  |
|  | Drive | - 4 x 2 double back wheels |  |  |  |
|  | Colour of chassis | - black RAL 9011 or equivalent |  |  |  |
|  | Colour of cabin | - white RAL 9010 or equivalent |  |  |  |
|  | Wheel base | - 3.600 mm - 3.700 mm |  |  |  |
|  | Air conditioner | - included |  |  |  |
|  | ABS | - included |  |  |  |
|  | Digital tachograph | - included |  |  |  |
|  | PTO (power - take off) at vehicle gearbox | - included |  |  |  |
|  | Gearbox | - 6 + 1 speed |  |  |  |
|  | Front axle load-bearing capacity | - 4450 kg |  |  |  |
|  | Back axle load-bearing capacity | - 8450 kg |  |  |  |
|  | Steel waste container - capacity | - 12 m³ + 1,3 m³ |  |  |  |
|  | Compacting ratio of compress plate | - 1:5 at waste mass of 85 kg/m³ |  |  |  |
|  | Compress plate working cycles | - manual cycle, single cycle, half-automatic synchronized with the emptying device |  |  |  |
|  | Binary compacting plate | - managed with 4 hydraulic cylinders |  |  |  |
|  | Binary compacting plate working pressure | - 160 bar |  |  |  |
|  | Electric blockage of engine with encoded key | - included |  |  |  |
|  | User manual in Croatian language | - included |  |  |  |
|  | Authorization of the manufacturer to sell the vehicle | - included |  |  |  |
|  | Authorized service in Split Dalmatia County | - included |  |  |  |
|  | Warranty for the vehicle | - 12 months |  |  |  |
|  | Unloading duration (full tank) | - 75 sec |  |  |  |
|  | Back door locking system during emptying or loading waste | - automatically regulated |  |  |  |
|  | Rotation of the engine for all functions | - automatically regulated |  |  |  |
|  | Emptying container device | - DIN / 120 - 1100 l , built in the back door according to norm EN 840-1,2,3 |  |  |  |
|  | Container tilting device protection | - according to norm EN 1510 - 1 |  |  |  |
|  | Managing device | - built in vertically at the back door with users guide in labels – or equivalent |  |  |  |
|  | Managing and control billboard built in cabin – system "touch screen" | - included |  |  |  |
|  | Emergency switch | - included |  |  |  |
|  | Self standing back platforms for operators with switch buttons | - included |  |  |  |
|  | Engine switch off system | - according to CE norms engine must be switched off when operator platforms are weighted and when the vehicle starts going backward |  |  |  |
|  | Three rotational orange lights | - included |  |  |  |
|  | Camera on the back with monitor in the cabin | - included |  |  |  |
| 2 | High pressure automatic wash device or equivalent (part of the cargo communal vehicle) |  |  |  |  |
|  | Parameter | Characteristics (minimal requirements) |  |  |  |
|  | High pressure flexible tube | - ø 3/8", length 20 m |  |  |  |
|  | High pressure wrist pump | - R 12 mm, Ø 3/8'' |  |  |  |
|  | Hydro motor for high pressure pump | - 100 bar |  |  |  |
|  | Board with compression | - Included |  |  |  |
|  | Measuring device | - Included |  |  |  |
|  | Electromagnetic valve | - Included |  |  |  |
|  | Flange for the pump | - Included |  |  |  |
|  | Valve with pointer of minimum water in container | - Included |  |  |  |
|  | Container volume | - 2 x 100 l |  |  |  |
|  | Warranty | 12 month |  |  |  |

# ANNEX II:	 TECHNICAL SPECIFICATIONS

| Specifications |  | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| Training | - To be provided for the staff of Komunalac d.o.o. Vela Luka - qualifications: - At least 1 year of experience in training of using the cargo communal vehicle - Fluency in both written and spoken Croatian or English |  |  |  |
|  | - training to be provided for 1 person |  |  |  |
|  | - duration of the training should be minimum 3 days |  |  |  |

DELIVERY/PLACE OF ACCEPTANCE

| Item | ARTICLE | Name of person responsible for provisional and final acceptance | Delivered/tested | Place of delivery |
| --- | --- | --- | --- | --- |
| Item 1 | Cargo communal vehicle for collection and transport of communal and recycled waste | Denis Andreis, Director | delivered/ installed tested | Komunalac d.o.o. Obala 3, no.19, 20270, Vela Luka, Croatia |

| Authorised Contact Person (for all Lots) | Delivery address (for all Lots) |
| --- | --- |
| Name: Boris Andreis – Minicipality Vela Luka Phone: + 385 20 812 609 Fax: + 385 20 813 033 E-mail: boris.andreis@velaluka.hr Working hours: 09:00 – 15:00 | Komunalac d.o.o. Obala 3, No.19, 20270, Vela Luka, Croatia |